CO2

SIMULATION EXPERIMENT – 5

Electromagnetic Simulation and	S-Parameter Characterization of	a Microwave Directional Coupler and Two-Way Power Divider.

- Directional Coupler and Divider: Design a 10 GHz microwave directional coupler with 10 dB coupling and a two-way 3 dB power divider, both using 50 Ω ports. Simulate from 8–12 GHz. At 10 GHz obtain S11, S21, S31 and the isolated-port response. Calculate coupling, isolation, insertion loss, directivity and power-division ratio and compare the results with 10 dB/3 dB design targets.

clc;

clear;

close all;

% EXPERIMENT 5A

% Directional Coupler and Two-Way Power Divider

f = linspace(8e9,12e9,401);

f0 = 10e9;

fGHz = f/1e9;

% Directional coupler parameters

Ctarget = 10;              % Coupling in dB

k0 = 10^(-Ctarget/20);

theta0 = asin(k0);

theta = theta0*(f/f0);

k = sin(theta);

S11 = zeros(size(f));

S21 = cos(theta);

S31 = 1i*k;

% Illustrative finite isolation model

% Assumed 35 dB isolation at 10 GHz

isoAmp = 10^(-35/20).*(1 + 8*abs(f-f0)/f0);

S41 = isoAmp;

% Coupler performance in dB

S11dB = 20*log10(max(abs(S11),1e-6));

S21dB = 20*log10(abs(S21));

S31dB = 20*log10(abs(S31));

S41dB = 20*log10(abs(S41));

Coupling = -S31dB;

Isolation = -S41dB;

Directivity = Isolation-Coupling;

InsertionLoss = -S21dB;

% Results at 10 GHz

[~,idx] = min(abs(f-f0));

Results_Coupler = table( ...

S11dB(idx),S21dB(idx),S31dB(idx),S41dB(idx), ...

Coupling(idx),Isolation(idx),Directivity(idx), ...

InsertionLoss(idx), ...

'VariableNames',{'S11_dB','S21_dB','S31_dB', ...

'S41_dB','Coupling_dB','Isolation_dB', ...

'Directivity_dB','InsertionLoss_dB'});

disp('DIRECTIONAL COUPLER RESULTS AT 10 GHz');

disp(Results_Coupler);

% Plot coupler S-parameters

figure;

plot(fGHz,S11dB,'LineWidth',1.5);

hold on;

plot(fGHz,S21dB,'LineWidth',1.5);

plot(fGHz,S31dB,'LineWidth',1.5);

plot(fGHz,S41dB,'LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Directional Coupler S-Parameters');

legend('S11','S21','S31','S41','Location','best');

ylim([-60 2]);

% Plot coupling, isolation, directivity and insertion loss

figure;

plot(fGHz,Coupling,'LineWidth',1.5);

hold on;

plot(fGHz,Isolation,'LineWidth',1.5);

plot(fGHz,Directivity,'LineWidth',1.5);

plot(fGHz,InsertionLoss,'LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Value (dB)');

title('Directional Coupler Performance');

legend('Coupling','Isolation','Directivity', ...

'Insertion Loss','Location','best');

% TWO-WAY POWER DIVIDER

% Idealized equal-split Wilkinson divider

% S11 = 0, S21 = S31 = 1/sqrt(2), S23 = 0

S11_div = zeros(size(f));

S21_div = ones(size(f))/sqrt(2);

S31_div = ones(size(f))/sqrt(2);

S23_div = zeros(size(f));

% Illustrative frequency-dependent mismatch

% Assumed return loss and output isolation degrade away from f0

x = (f-f0)/f0;

S11_div = 0.01 + 0.08*abs(x);

S21_div = (1/sqrt(2))*cos(pi*x/2);

S31_div = S21_div;

S23_div = 0.01 + 0.08*abs(x);

S11divdB = 20*log10(abs(S11_div));

S21divdB = 20*log10(abs(S21_div));

S31divdB = 20*log10(abs(S31_div));

S23divdB = 20*log10(abs(S23_div));

% Results at 10 GHz

Results_Divider = table( ...

S11divdB(idx),S21divdB(idx),S31divdB(idx), ...

S23divdB(idx), ...

'VariableNames',{'S11_dB','S21_dB','S31_dB','S23_dB'});

disp('TWO-WAY POWER DIVIDER RESULTS AT 10 GHz');

disp(Results_Divider);

% Plot divider S-parameters

figure;

plot(fGHz,S11divdB,'LineWidth',1.5);

hold on;

plot(fGHz,S21divdB,'LineWidth',1.5);

plot(fGHz,S31divdB,'LineWidth',1.5);

plot(fGHz,S23divdB,'LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Two-Way Power Divider S-Parameters');

legend('S11','S21','S31','S23','Location','best');

ylim([-60 2]);

% Power division ratio at 10 GHz

P2 = abs(S21_div(idx))^2;

P3 = abs(S31_div(idx))^2;

PowerRatio = P2/P3;

disp('Power division ratio P2/P3:');

disp(PowerRatio);

- Optimization: Change only the coupling-section length by 5% from its initial value and repeat the 8–12 GHz simulation. Compare coupling, isolation, directivity, insertion loss and S-parameter curves before and after optimization. Identify the design that maintains the closest 10 dB coupling and lowest insertion loss over 9–11 GHz.

clc;

clear;

close all;

% EXPERIMENT 5B

% Coupling Section Length Optimization

f = linspace(8e9,12e9,401);

f0 = 10e9;

fGHz = f/1e9;

% Design specifications

Ctarget = 10;

k0 = 10^(-Ctarget/20);

theta0 = asin(k0);

% Initial length and modified length

L0 = 1;               % Normalized initial length

Lnew = 1.05*L0;       % Increase length by 5%

% ORIGINAL DESIGN

theta1 = theta0*(f/f0)*(L0/L0);

k1 = sin(theta1);

S11_1 = zeros(size(f));

S21_1 = cos(theta1);

S31_1 = 1i*k1;

iso1 = 10^(-35/20).*(1+8*abs(f-f0)/f0);

S41_1 = iso1;

C1 = -20*log10(abs(S31_1));

I1 = -20*log10(abs(S41_1));

D1 = I1-C1;

IL1 = -20*log10(abs(S21_1));

% MODIFIED DESIGN: LENGTH INCREASED BY 5%

theta2 = theta0*(f/f0)*(Lnew/L0);

k2 = sin(theta2);

S11_2 = zeros(size(f));

S21_2 = cos(theta2);

S31_2 = 1i*k2;

% Illustrative isolation model with length mismatch penalty

iso2 = 10^(-35/20).*(1+8*abs(f-f0)/f0 ...

+ 20*abs(Lnew/L0-1));

S41_2 = iso2;

C2 = -20*log10(abs(S31_2));

I2 = -20*log10(abs(S41_2));

D2 = I2-C2;

IL2 = -20*log10(abs(S21_2));

% S-parameter magnitudes in dB

S11dB1 = 20*log10(max(abs(S11_1),1e-6));

S21dB1 = 20*log10(abs(S21_1));

S31dB1 = 20*log10(abs(S31_1));

S41dB1 = 20*log10(abs(S41_1));

S11dB2 = 20*log10(max(abs(S11_2),1e-6));

S21dB2 = 20*log10(abs(S21_2));

S31dB2 = 20*log10(abs(S31_2));

S41dB2 = 20*log10(abs(S41_2));

% Find 10 GHz results

[~,idx] = min(abs(f-f0));

Results = table( ...

["Original";"Length +5%"], ...

[C1(idx);C2(idx)], ...

[I1(idx);I2(idx)], ...

[D1(idx);D2(idx)], ...

[IL1(idx);IL2(idx)], ...

'VariableNames',{'Design','Coupling_dB', ...

'Isolation_dB','Directivity_dB','InsertionLoss_dB'});

disp('COMPARISON AT 10 GHz');

disp(Results);

% Evaluate designs from 9 to 11 GHz

band = fGHz >= 9 & fGHz <= 11;

MeanCouplingError1 = mean(abs(C1(band)-Ctarget));

MeanCouplingError2 = mean(abs(C2(band)-Ctarget));

MeanIL1 = mean(IL1(band));

MeanIL2 = mean(IL2(band));

BandResults = table( ...

["Original";"Length +5%"], ...

[MeanCouplingError1;MeanCouplingError2], ...

[MeanIL1;MeanIL2], ...

'VariableNames',{'Design','MeanCouplingError_dB', ...

'MeanInsertionLoss_dB'});

disp('AVERAGE PERFORMANCE FROM 9 TO 11 GHz');

disp(BandResults);

% Plot S-parameters before and after modification

figure;

subplot(2,2,1);

plot(fGHz,S11dB1,'b',fGHz,S11dB2,'r--','LineWidth',1.5);

grid on;

title('S11 - Reflection');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('Original','Length +5%');

subplot(2,2,2);

plot(fGHz,S21dB1,'b',fGHz,S21dB2,'r--','LineWidth',1.5);

grid on;

title('S21 - Through Port');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('Original','Length +5%');

subplot(2,2,3);

plot(fGHz,S31dB1,'b',fGHz,S31dB2,'r--','LineWidth',1.5);

grid on;

title('S31 - Coupled Port');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('Original','Length +5%');

subplot(2,2,4);

plot(fGHz,S41dB1,'b',fGHz,S41dB2,'r--','LineWidth',1.5);

grid on;

title('S41 - Isolated Port');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('Original','Length +5%');

% Compare coupling, isolation, directivity and insertion loss

figure;

subplot(2,2,1);

plot(fGHz,C1,'b',fGHz,C2,'r--','LineWidth',1.5);

hold on;

yline(10,'k:');

grid on;

title('Coupling');

xlabel('Frequency (GHz)');

ylabel('Coupling (dB)');

legend('Original','Length +5%','10 dB target');

subplot(2,2,2);

plot(fGHz,I1,'b',fGHz,I2,'r--','LineWidth',1.5);

grid on;

title('Isolation');

xlabel('Frequency (GHz)');

ylabel('Isolation (dB)');

legend('Original','Length +5%');

subplot(2,2,3);

plot(fGHz,D1,'b',fGHz,D2,'r--','LineWidth',1.5);

grid on;

title('Directivity');

xlabel('Frequency (GHz)');

ylabel('Directivity (dB)');

legend('Original','Length +5%');

subplot(2,2,4);

plot(fGHz,IL1,'b',fGHz,IL2,'r--','LineWidth',1.5);

grid on;

title('Insertion Loss');

xlabel('Frequency (GHz)');

ylabel('Insertion Loss (dB)');

legend('Original','Length +5%');

% Identify the design closest to the target

if MeanCouplingError1 <= MeanCouplingError2 ...

&& MeanIL1 <= MeanIL2

disp('Original design gives the better combined result.');

elseif MeanCouplingError2 <= MeanCouplingError1 ...

&& MeanIL2 <= MeanIL1

disp('Modified design gives the better combined result.');

else

disp('Trade-off exists; compare coupling error and insertion loss.');

end

SIMULATION EXPERIMENT – 6

Simulation-Based	Investigation	of	Microwave	Semiconductor	Devices	and Frequency-Conversion Characteristics of a Microwave Mixer

Objectives of this Experiment:

1.	To develop simulation models representing the operating characteristics of selected microwave semiconductor devices.

2.	To analyze the electrical characteristics and frequency-dependent behavior of Gunn, Schottky or PIN diode-based microwave structures.

3.	To simulate frequency conversion in a microwave mixer and identify the generated frequency components.

4.	To evaluate conversion characteristics and identify suitable operating conditions for microwave applications.

Experimental Procedure

A.	Mixer Frequency Conversion: Set fRF = 10 GHz, fLO = 9.5 GHz, PRF = −10 dBm and PLO = +7

dBm with a 50 Ω IF load. Obtain the output spectrum. Calculate the theoretical IF frequency |10−9.5| =

0.5 GHz and sum frequency 19.5 GHz. Identify the simulated components and calculate the percentage frequency deviation.

clc;

clear;

close all;

% EXPERIMENT 6A

% Microwave Mixer Frequency Conversion

fRF = 10;       % RF frequency in GHz

fLO = 9.5;      % LO frequency in GHz

PRF = -10;      % RF power in dBm

PLO = 7;        % LO power in dBm

% Calculate frequency components

fIF = abs(fRF-fLO);

fSUM = fRF+fLO;

fprintf('MICROWAVE MIXER FREQUENCY CONVERSION\n');

fprintf('RF Frequency = %.2f GHz\n',fRF);

fprintf('LO Frequency = %.2f GHz\n',fLO);

fprintf('IF Frequency = %.2f GHz\n',fIF);

fprintf('Sum Frequency = %.2f GHz\n',fSUM);

% Illustrative spectrum levels

freq = [fIF fLO fRF fSUM];

power = [-18 PLO PRF -25];

figure;

stem(freq,power,'filled','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Power (dBm)');

title('Microwave Mixer Frequency Spectrum');

legend('Frequency Components');

B.Conversion Loss: Using the same mixer, record the IF output power at 0.5 GHz. Calculate conversion loss CL = PRF − PIF and compare it with the expected passive-mixer range of approximately 6–10 dB. Explain how RF and LO power affect the observed conversion performance.

clc;

clear;

close all;

% EXPERIMENT 6B

% Conversion Loss of Microwave Mixer

PRF = -10;          % RF input power in dBm

PLO = 7;            % LO power in dBm

PIF = -18;          % Assumed IF output power in dBm

% Conversion loss

CL = PRF-PIF;

fprintf('MICROWAVE MIXER CONVERSION LOSS\n');

fprintf('RF Input Power = %.2f dBm\n',PRF);

fprintf('LO Power = %.2f dBm\n',PLO);

fprintf('IF Output Power = %.2f dBm\n',PIF);

fprintf('Conversion Loss = %.2f dB\n',CL);

if CL >= 6 && CL <= 10

fprintf('Conversion loss is within the expected range.\n');

else

fprintf('Conversion loss is outside the expected range.\n');

end

% Plot conversion loss

figure;

bar(CL);

grid on;

ylabel('Conversion Loss (dB)');

title('Microwave Mixer Conversion Loss');

ylim([0 12]);

% Theoretical frequency deviation

fRF = 10;

fLO = 9.5;

fIF = abs(fRF-fLO);

fSUM = fRF+fLO;

fprintf('Theoretical IF Frequency = %.2f GHz\n',fIF);

fprintf('Theoretical Sum Frequency = %.2f GHz\n',fSUM);